UNIT-I INTRODUCTION

- Computer architecture

- Refers to those attributes of a system visible to a programmer or, put another way, those attributes that have a direct impact on the logical execution of a program.

- Examples of architectural attributes include the instruction set, the number of bits used to represent

various data types (e.g., numbers, characters), I/O mechanisms, and techniques for addressing memory.

- Computer organization

- Refers to the operational units and their interconnections that realize the architectural specifications.

- Organizational attributes include those hardware details transparent to the programmer, such as control signals; interfaces between the computer and peripherals; and the memory technology used.

- For example, it is an architectural design issue whether a computer will have a multiply instruction. It is an organizational issue whether that instruction will be implemented by a special multiply unit or by a mechanism that makes repeated use of the add unit of the system.

- The components from which computers are built, i.e., computer organization.

- In contrast, computer architecture is the science of integrating those components to achieve a level of functionality and performance.

Basic Terminology

  - Input Whatever is put into a computer system.

  - Data Refers to the symbols that represent facts, objects, or ideas.

  - Information The results of the computer storing data as bits and bytes; the words, 				numbers, sounds, and graphics.

  - Output Consists of the processing results produced by a computer.

  - ProcessingManipulation of the data in many ways.

  - Memory Area of the computer that temporarily holds data waiting to be 					processed, stored, or output.

  - Storage Area of the computer that holds data on a permanent basis when it is not 				immediately needed for processing.

- Basic Terminology

- Assembly language program (ALP) –Programs are written using mnemonics

- Mnemonic –Instruction will be in the form of English like form

- Assembler –is a software which converts ALP to MLL (Machine Level Language)

- HLL (High Level Language) –Programs are written using English like statements

- Compiler -Convert HLL to MLL, does this job by reading source program at once

- Basic Terminology

- Interpreter –Converts HLL to MLL, does this job statement by statement

Operating system –Collection of routines responsible for controlling and coordinating all 					the activities in a computer system

Computers have two kinds of components:

- Hardware, consisting of its physical devices (CPU, memory, bus, storage devices ...)

- Software, consisting of the programs it has (OS, applications, utilities, ...)

- FUNCTIONAL UNITS OF COMPUTER

- Input Unit

- Output Unit

- Central processing Unit (ALU and Control Units)

- Memory

- Bus Structure

Program Execution

Translation: The entire high-level program is translated into an equivalent machine language program. Then the machine language

Program is executed.

Interpretation: Another program reads the high level program instructions one-by-one and executes a equivalent series of

Machine language instructions.

Program translation uses a collection of tools to perform the translation

_ Compiler: Translates high level language programs into a lower level language often called object code.

_ Assembler: Translates assembly language instructions into object code.

_ Linker: Combines collections of object code into a single executable machine language program.

Structure: The way in which the components are interrelated

Function: The operation of each individual component as part of the structure

There are four main functions of a computer:

- Data processing

- Data storage

- Data movement

- Control

MAIN STRUCTURAL BLOCKS/PARTS:

Central Processing Unit (CPU): Controls the operation of the computer and performs its data 										processing 	functions. Often simply referred to as processor.

Main Memory: Stores data.

I/O: Moves data between the computer and its external environment.

System Interconnection: e.g. BUS for communication among CPU, main memory, and I/O.The major 								structural components of a CPU are:

Control Unit (CU): Controls the operation of the CPU and hence the computer.

Arithmetic and Logic Unit (ALU): Performs computer’s data processing functions.

Register: Provides storage internal to the CPU.

CPU Interconnection: communication among the control unit, ALU, and register.

Stored Program Concept

four general purpose registers

There are four general purpose registers which are AX, BX, CX and DX.

•

AX, which is named the accumulator. It is used for all input/output operations and some arithmetic operations. For example, multiply, divide and translate instructions assume the use of AX.

•

BX, which is named the base register. It can be used as an address register.

•

CX, which is the count register. It is used by instructions which require to count. Typically it is used for controlling the number of times a loop is repeated and in bit shift operations.

•

DX, which is the data register. It is used for some input/output and also when multiplying and dividing.